Step 11. Measure Position 3

Within this section, you will take the final set of measurements of the excavator’s linkages. This process is the same as positions 1 and 2 in step 8. Utilize Siteworks and Hexcal to calculate the necessary measurements. Enter these measurements within the Calibration Readings sections of the Measure Up menu.

This section, similar as before, is to 2D measurement, but also a portion will be to the 3D measure-up.

Therefore, as far as necessary the Points CT1, CT2, A, B1, B, G, F, D and ML, MR will be measured and entered into Earthworks in this section. The Position for the COR measurements will be taken in a later step and used to calculate the Center of Rotation of the machine.

Note: Do not tear down the UTS as it will be required in a following step