CAN

Troubleshooting CAN

- Check terminators are present and each has 120 ohm resistance

- Remove terminator and check the other through the length of the harness

- Check resistance is 60 ohms over the CAN bus between CAN High and CAN Low

Note: Only check resistance when the power off

- Check continuity between CAN High pins

- Check continuity between CAN Low pins

- Disconnect all components and reconnect one at a time