Earthworks Components

TD540 Display

- 10.1” Sunlight Readable LCD Touchscreen

- Android Operating System

- Quick Release RAM mounting

- Secure platform for portability of grade control option keys

- Can operate in Landscape or Portrait mode

- Light bars integrated within operation screens

- Front facing USB for easy FW upgrade and synchronization of design and productivity data

- Power down delay configurable by user

- Split screen viewing configurable by user

- Can contain all Earthworks system licensing for both machine types

TD520 and TD510

- TD520 is legacy 10.1” display (replaced by TD540)

- TD510 is a smaller format premium machine display for Trimble Earthworks.

- The 7-inch, daylight readable unit is smaller in physical dimensions when compared to the 10.1 inch Trimble TD520 or TD540 Display.

- Display utilizes a different connector than TD540. Adapters are required to swap display models between the TD540 and legacy displays

EC520 ECM

- Integrated Inertial Measurement Unit (IMU) body sensor with 6 degrees of freedom (3 Axis)

- 3 CAN ports for Machine CAN, HMI devices and grade control sensor connectivity

- 2 Ethernet ports for displays and radios

- Grade control system power management

- IP67 rated against dust, water, and shock

GS520 Grade Sensor

- Integrated Inertial Measurement Unit (IMU) grade sensor with 6 degrees of freedom(3 Axis)

- Suitable for harsh vibration environments (HEX bucket and dozer blades)

- Mount directly to linkage; no shock mounting required

- IP67 rated against dust, water, and shock

VM510 Valve Module

- Translates CAN messages from EC520 and sends to Machine Implement ECM

- Directly controls aftermarket hydraulic valve if one is installed.

- IP67 rated against dust, water, and shock

MS996 Receiver

- Advanced RTK engine for faster initialization times when satellite lock is lost and enhanced performance near obstructions

- Same form factor as legacy MS995s with improved performance

- Support for GPS GLONASS, Galileo, BeiDou

- 3 LED indicators that provide instant operational feedback

- Tracks up to 44 Satellites

- Waterproof (up to 5PSI)

- Hardened against dust intrusion and shock

MS976 Receiver

- Advanced RTK engine for faster initialization times when satellite lock is lost and enhanced performance near obstructions

- Same form factor as legacy MS975s with improved performance

- Support for GPS GLONASS, Galileo, BeiDou

- 3 LED indicators that provide instant operational feedback

- Tracks up to 44 Satellites

- Waterproof (up to 5PSI)

- Hardened against dust intrusion and shock

SNR930 Radio

- 900 or 450 mhz Receive only radio

- LED provides information on radio status

- IP67 rated against dust, water, and shock

- Receives signal broadcast by base station and transmits to MS9x6 receivers over wiring harnesses

LR510 Laser Receiver

2D Machine Control

3D Machine Control – Single GNSS

3D Machine Control – Dual GNSS

3D Machine Control – Single UTS

2D & 3D Machine Control – Additional Options