Installer Responsibilities

Before Installation: Bench Test Responsibilities

1. Confirm the latest revision of installation instructions, ensuring allocated time for installation.

2. Schedule location, installer, welder

3. Test all CAN/Onboard Devices on a test bench

4. Load and Verify all codes from Virtual Warehouse

5. Add serial Number and firmware version on service database MTP

6. Ensure all harnessing and components are properly inspected and tested.

Installer/Tech Responsibilities:

1. Visually inspect the machine to ensure it’s in good working condition, no fault codes, no worn linkages, and is safe to operate for the customer/operator!

2. Ensure you have the proper tooling to complete the installation. It is always wise to familiarize yourself with the installation manual and see if newer revisions are available.

3. Ensure the weldments are in the exact locations as outlined in the installation documentation. If this is off significantly, it will cause performance issues.

4. Install the components and harness in a manner in which the Trimble components are serviceable and is easy to access for troubleshooting purposes.