Horizontal Guidance

You can receive horizontal guidance relative to the following design features:

- Lines within Office Designs or Infield Designs

- Design surface boundaries

- Setting a horizontal offset moves the horizontal guidance away from the selected line. You

- Add or subtract an offset square to the selected horizontal line.

Note- Design must have linework present for this functionality to be available

1. Access the Horizontal Guidance screen.

Method 1

Touch and hold the horizontal offset field in the guidance bar

2. Select a line for horizontal guidance and select Apply.

Method 2

Alternatively, you can also select a line for horizontal guidance from the context menu in the work screen:

1) From the plan view guidance pane, press and hold anywhere near the line you want to select. A context menu appears.

2) Tap Select 2D Line and select an available line.

3) Tap Horizontal Guidance.

Apply a horizontal offset:

1. Once inside the Horizontal Guidance Menu, Select a line to guide to.

2. Enter the value of the offset or tap the up or down buttons to increase or decrease the value.

Note- To save this as a memory, tap Save.

3. Tap Apply